COPDOCK and WASHBROOK PLAY AREAS SAFETY INSPECTION REPORT 2026

SUMMARY OF FINDINGS & RECOMMENDATIONS

| LOCATION | RECOMMENDATION | RISK |
| --- | --- | --- |
| Play Area, Village Hall | Site General: Finding: The area around the play area has eroded and may become slippery. Action: Reinstate the eroded area. Sign: Finding: The sign has been attached to the metal gate with cable ties which have sharp projecting ends. Action: Replace with more appropriate fixing method. Metal-bar gate: Finding: There are projecting bolt thread(s) present. Action: Remove excess thread length and deburr or provide cap. Finding: Cable ties are present and there are sharp projecting ends. Action: Remove Gate: Finding: The wooden gate has openings less than 12mm that could trap or crush fingers. Action: Provide a 12mm gap throughout the full range of motion at both sides of the gate. Finding: The area around the gate has eroded and may become slippery. Action: Reinstate the eroded area. Bench: Finding: There are trip hazards at the edges of the surface. Action: Reinstate surface levels to remove trip hazard. See Saw: Finding: There are gaps opening between the grass mat tiles. Action: Provide additional tiles or secure/reinstate surface. Finding: The paint is flaking off the metal work. Action: Rub down and repaint. 1 Bay, 2 Seat: Finding: The grass mats are silted up, the soil is compacted and the area will become very slippery when the soil is wet. Action: Reinstate as required. Finding: The edges of the grass mat surface are raised/lifting above the surrounding surface. Action: Secure the edges of grass mats as per the API Code of Practice to remove the trip hazards. Finding: There is moderate damage to the surface. Action: Repair the damaged areas of surfacing. Finding: There is algae or moss on the surface of the seats. Action: Clean and treat appropriately. Finding: The paint is flaking off the metalwork. Action: Rub down and repaint. Fence, Steel Mesh: Finding The adjacent foliage is overhanging. Action: Cut back and maintain. | Low Low Low Low Low Low Low Low Low Low Low Low Low Low Low |
| Play Area, Fen View | Site General: Finding There is no physical barrier between the road and the play area. Action: Ensure a full and sufficient risk assessment is in place and review regularly. Finding: There is weed/vegetation growth within the surface. Action: Remove weed/vegetation growth. Finding: There is a damaged and unsecured domestic football goal on site. Action: Remove. Litter Bin: Finding: The post is loose in its foundations. Action: Reinstall item to secure. Bench: Finding: The paintwork on this item has been damaged or worn exposing the metal underneath which is rusty/corroding. Action: Treat affected areas and repaint. Rope Bridge: Finding: The paintwork has been damaged or worn exposing metal underneath which is rusting/corroding. Action: Treat affected areas and repaint. Finding: The connecting links fail the requirements. Action: Replace with compliant links. Finding: There are missing inserts creating finger entrapment/s in the framework. Action: Replace the missing parts. Chain Traverse: Finding Bolt cap covers are missing or damaged on the framework. Action: Replace missing or damaged bolt cap covers. 2-Bay: Finding: There is moderate damage to the surface. Action: Repair the damaged areas of surfacing. Finding: There is algae or moss on the surface of the seats. Action: Clean and treat appropriately. Multi Play Junior: Finding: Bolt cap covers are missing or damaged on the slide guard. Action: Replace missing or damaged bolt cap covers. Finding: There is/are fixings missing on the post. Action: Replace all missing fixings. Finding: A number of fixings have worked loose on the rope net. Action: Secure all loose fixings. Rotating Net: Finding: The paint is flaking off the metalwork. Action: Rub down and repaint. Finding: There is/are steep gradients along the edges of the synthetic surface. Action: Build up surrounding surface levels to remove trip hazard. Free Standing Slide: Finding: There are projecting bolt thread(s) present. Action: Remove excess thread length and deburr or provide caps. Finding: The paint is flaking off the metalwork. Action: Rub down and repaint. Roundabout: Finding: The paint is flaking off the metalwork. Action: Rub down and repaint. Finding: The bearing is worn / damaged and the item is not operating correctly. Action: Repair/replace | Low Low Low Low Low Low Low Low Low Low Low Low Low Low Low Low Low Low Low Low |

Mill Lane Play Area has not been included in light of the planned upgrade improvements 2026.